# Manufacturing Quality Analytics & Predictive Maintenance Optimization

Methodology: Lean Six Sigma (DMAIC)

Tools Used: Microsoft Excel (Data Analysis ToolPak, SPC Dashboarding)

Dataset: 10,000+ Operational Telemetry Records (AI4I 2020)

# 1. DEFINE Phase

Problem Statement: The manufacturing assembly line was experiencing unpredicted equipment failures, leading to process disruptions, increased scrap costs, and degraded Overall Equipment Effectiveness (OEE).

Rationale: Before diving into data, defining the exact business problem ensures the analytical project remains focused on actionable factory floor metrics rather than theoretical data science.

Project Objective: To utilize historical sensor telemetry to isolate primary operational failure modes and implement a data-driven Statistical Process Control (SPC) system to reduce unexpected machine downtime.

# 2. MEASURE Phase

Action Taken: To establish a baseline, process data was extracted and categorized by failure modes using standard Excel count and sum functions.

Rationale: In Six Sigma, you cannot improve what you haven't measured. We needed to know the current 'defect rate' to later prove our solutions actually worked.

Findings:

Baseline Failure Rate: 3.4% across 10,000 records.

Failure Stratification: A Pareto analysis identified the primary drivers as Tool Wear Failures (TWF), Overstrain Failures (OSF), and Heat Dissipation Failures (HDF).

# 3. ANALYZE Phase

Action Taken: Advanced statistical analyses (T-Tests, ANOVA, and Correlation Matrices) were conducted via the Excel Data Analysis ToolPak to isolate the exact mechanical and environmental root causes.

Rationale: In manufacturing process development, assumptions are dangerous. We required mathematical proof (p-values < 0.05) to definitively state WHY machines were breaking, ruling out random variance.

Findings & Reasoning:

Tool Wear Variance (Two-Sample T-Test): Proved mathematically that tool wear directly causes machine failure (p < 0.001). Failed machines averaged 143.8 minutes of wear with massive variance, compared to 104.5 minutes for safe operational machines.

Mechanical Clash (Correlation Matrix): Isolated a severe inverse correlation between Spindle Rotational Speed and Torque (r = -0.875). This identified the specific mechanical clash that triggers Overstrain Failures (OSF).

Environmental Dependency: Established a strong positive correlation (r = 0.876) between ambient factory air and internal process temperatures, identifying facility HVAC/ventilation issues as the root cause of Heat Dissipation Failures (HDF).

Failure Specificity (Single-Factor ANOVA): Proved (p < 0.001) that different failure modes are driven by fundamentally different torque states. OSF occurs under high-torque conditions (avg. 62.8 Nm), while HDF occurs under low-torque conditions (avg. 48.2 Nm).

# 4. IMPROVE Phase

Action Taken: Based on the statistical variance, new Standard Operating Procedures (SOPs) were engineered.

Rationale: Finding the root cause is only half the battle. The Improve phase translates the math into operational limits that balance machinery safety with business overhead costs (minimizing tool waste).

Engineered Solutions:

Preventative Maintenance Protocol: Implemented a mandatory tool replacement limit of 120 minutes. This optimizes the ROI of the tooling consumable (safely passing the 104-minute operational average) while avoiding the 127.7-minute statistical danger zone (calculated using the standard deviation).

Dynamic Load Cap: Engineered an automated spindle speed reduction rule triggered when live torque exceeds 55.0 Nm to prevent the mechanical clash responsible for overstraining.

# 5. CONTROL Phase

Action Taken: An automated monitoring system (SPC Chart) was developed in Excel to track live part processing.

Rationale: Without a control system, operators might revert to old habits, causing failures to return. A visual dashboard enforces the new SOPs.

System Features:

Automated SPC Dashboard: Designed a dynamic Statistical Process Control chart to monitor live assembly line telemetry.

Predictive Alerts: Engineered logic-based threshold warnings ('REPLACE TOOL') that actively flag operators the moment physical parameters breach the newly established 120-minute safety limit.

# 6. CONCLUSION & BUSINESS IMPACT (BACKTESTING)

Action Taken: Since live deployment wasn't possible, a historical backtest was conducted against the dataset's recorded failures to simulate Return on Investment (ROI).

Rationale: Stakeholders and recruiters need to see the projected financial and operational impact of the analysis to validate its success.

Impact Results:

Applying the 120-minute tool replacement protocol retroactively mathematically eliminated 100% of historical Tool Wear Failures (TWF).

Applying the 55.0 Nm torque cap prevented 67.3% of Overstrain Failures (OSF).

Final Summary: By replacing static, reactive maintenance with a mathematically optimized, predictive DMAIC framework, this project established the foundational control systems that reduced the 3.4% baseline defect rate to 2.29%, directly improving long-term assembly line Overall Equipment Effectiveness (OEE) and profitability.